2.For the BJT amplifier circuit shown in Figure 2, where Vc=20V,β=200, VBEQ=0.7V,

a. Find the value of IBQ,ICQ。

b. Draw the small-signal equivalent circuit of the amplifier.

c. Find the voltage gain.

d. Find the input resistance,ri .

e. Find the output resistance, ro.

3. The circuit shown in Figure 3 is a common-drain (or source follower) amplifier. The

transistor parameters, and component values are: RL = 1 kΩ and R1 = R2 =2 MΩ. KP=50μA/V2, Vto=1V, L=2 μm, W=160 μm.

- Find the value for RS to achieve IDQ = 10 mA.

- Draw the AC small-signal equivalent circuit.

- Determine the values of the input resistance Ri, the output resistance R0, and voltage gain Av.

4. The circuit composed of ideal operational amplifiers is shown in Figure 4,

a. Derive the expression of V0.

b. When R1=R2=R3=Rf，V0=?

c. What is the feedback type of Rf?

7. The differential amplifiers is shown below，where β =50, Ube =0.7V, input voltage ui1=6mV, ui2=4mV ,Ucc=6V, Ee=-6V, RB=10 kΩ , Rc=5 kΩ,RE=5 kΩ;

a. Find the values of IBQ,ICQ.

b. Decompose ui1and ui2to common-mode signals uic1and uic2as well as differential signals uid1and uid2 .

c. Calculate the single-ended outputs uod1and uod2for differential signals;

d. If the gain of common mode signal for single-ended outputs is Ac=− Rc/2RE ，calculate the single-ended outputs uo1and uo2；

e. Calculate the common-mode output uocand differential output uod.

Answer：

b.

C.

d.

E.

3.

A.

B.

C.

4.

5.

6.